Table of Contents

# Data dictionary

Source column identifiers are Spanish, exactly as the publisher issues them. This maps each to its meaning and unit. Derived fields are marked [derived] with the rule that produces them.

## Units, once and for all

| Quantity | Stored unit | Note |
| --- | --- | --- |
| Oil | m³ | 1 m³ = 6.28981 bbl — re-derived from the publisher's own redundant columns, not taken from a reference |
| Gas | 10³ m³ | "Mm³" in Argentine usage means thousands, not millions. 1 × 10³ m³ = 35.3147 Mcf |
| Water, injection | m³ |  |
| tef | hours | effective production time in the month; ceiling is 744 h |
| Depth, lateral | m |  |
| Proppant | tonnes | national + imported, summed |
| Coordinates | degrees | WGS 84 (EPSG:4326), confirmed by decoding the geometry's SRID |

Nothing is ever stored converted. Oilfield units are a display-time toggle, so there is exactly one number of record.

## wells — one row per idpozo

idpozo is a wellbore × producing formation, not a wellbore. A well producing from two formations appears twice. Every "well count" on the site counts idpozo.

| Column | Meaning |
| --- | --- |
| idpozo | integer identity of the wellbore × formation pair; the primary key |
| sigla | human-readable well code (e.g. YPF.BLO.x-8); a label, and mutable |
| formprod | producing formation — the one encoded in the identity |
| formacion | geological formation as reported |
| area, cod_area | permit / concession area |
| yacimiento | field |
| cuenca | sedimentary basin |
| provincia | province |
| clasificacion, subclasificacion | regulatory classification (Exploración, Desarrollo …) |
| tipo_recurso | CONVENCIONAL / NO CONVENCIONAL / No informado / SIN RESERVORIO |
| sub_tipo_recurso | SHALE / TIGHT / No informado |
| cota | surface elevation, m a.s.l. |
| profundidad | final well depth, m |
| coordenadax / coordenaday | longitude / latitude, WGS 84 degrees |
| geom | geometry as WKB, SRID 4326 |
| adjiv_fecha_inicio_perf … _fin_term | spud, drilling end, completion start, completion end |
| pet_inicial, gas_inicial, agua_inicial | initial test rates (m³/d, 10³ m³/d, m³/d) |
| has_production | false for registry wells that never produced |

## monthly_production — one row per (idpozo, fecha)

| Column | Meaning |
| --- | --- |
| fecha | measurement month, first day of month |
| prod_pet | oil produced in the month, m³ |
| prod_gas | gas produced, 10³ m³ |
| prod_agua | water produced, m³ |
| iny_agua, iny_gas, iny_co2, iny_otro | injection volumes |
| tef | Tiempo Efectivo de Producción — effective production hours |
| vida_util | declared remaining useful life, months |

Null is not zero. Gaps in a well's series carry NULL measurements; a declared zero is a real zero. About 0.8% of well-months are NULL. Statistics skip NULLs and report them separately.

## well_operator_history / well_events

| Column | Meaning |
| --- | --- |
| idempresa | operator code — VARCHAR, not an integer (Z001, APEA) |
| empresa | operator display name |
| valid_from, valid_to | inclusive month bounds of one operator run; valid_to NULL = still current |
| tipoestado | operational state (Extracción Efectiva, Parado Transitoriamente …) |
| tipoextraccion | lift method (Bombeo Mecánico, Surgente …) |
| tipopozo | well type by fluid (Petrolífero, Gasífero, Inyección de Agua) — this is not trajectory |

## Fracture table (Adjunto IV) — one row per fracture job

Joined to production on idpozo. Covers 4,604 of 85,417 wells (76% of unconventional wells, ~1% of conventional).

| Column | Meaning |
| --- | --- |
| longitud_rama_horizontal_m | lateral length, m — the source of trajectory |
| cantidad_fracturas | number of fracture stages |
| tipo_terminacion | completion type (Punzado = plug-and-perf …) |
| arena_bombeada_nacional_tn / _importada_tn | proppant pumped, tonnes, domestic / imported |
| agua_inyectada_m3 | fluid volume, m³ |
| presion_maxima_psi | maximum treating pressure, psi |
| potencia_equipos_fractura_hp | pump power, hp |
| fecha_inicio_fractura / _fin_ | job start / end |

## Derived fields

| Field | Rule |
| --- | --- |
| trajectory [derived] | no fracture record → Unknown; lateral ≥ 500 m → Horizontal; else Vertical. The 500 m cut sits inside an almost empty corridor in the distribution — only 20 of 4,604 wells fall between 150 m and 600 m — so the classification is insensitive to it. Never imputed for wells without a record. |
| lateral_m [derived] | max across jobs — a property of the wellbore, so it does not sum |
| stages, proppant_t, frac_water_m3 [derived] | sum across jobs — consumed per job, so they do |
| max_pressure_psi [derived] | max across jobs — an observed peak |
| proppant_kg_per_m [derived] | proppant_t × 1000 / lateral_m; NULL when lateral is 0 |
| stage_spacing_m [derived] | lateral_m / stages |
| gor_m3_m3 [derived] | cum_gas × 1000 / cum_oil (the ×1000 converts 10³ m³ → m³) |
| first_prod_declared [derived] | first-production month from the official padrón |
| first_prod_month [derived] | first month with non-zero oil or gas in this dataset |
| operator_asof [derived] | operator holding the well in that month, by range join |
| well_fluid_asof [derived] | tipopozo in force that month, by ASOF join (events record only transitions) |
| operator_latest [derived] | most recent operator — a different question, and a different answer |
| tef (in prod_monthly) [derived] | clamped to the hours available in the month; five source rows exceed it, worst case 3,058 h |

## Statistical conventions

- p10 is the LOW value. p10 < p50 < p90. Stated on the page itself, because the reserves convention runs the other way.

- Unit of observation is always declared. A statistic over well-months and a statistic over wells answer different questions; the Statistics page names which one is on screen.

- Well counts are per month. A well recurs in every month it produces, so counts may be summed across categories within a month but never along time.